Black Face Pro Reverb

| Year: | 1965-1967 |
| --- | --- |
| Model: | Pro Reverb |
| Circuit: | AA165 |
| Config: | Combo |
| Control Panel: | Black, forward facing w/ white labels |
| Front Conrol Layout: | Normal: In, In, Bright Sw, Vol, Treb, Bass - Vibrato: In, In, Bright Sw, Vol, Treb, Bass, Reverb, Speed, Intensity - Pilot Lamp |
| Rear Conrol Layout: | AC Outlet, Polarity Sw, Fuse (2A), Power Sw, Stby Sw, Speaker, Ex. Speaker, Vibrato Jack, Reverb Jack, Reverb Out, Reverb In |
| Knobs: | Black skirted w/ chrome center, numbered 1 - 10 |
| Cabinet: | 19½" x 26½" x 9½" (49.5 x 67.3 x 24.1 cm) |
| Cab Covering: | Black Tolex |
| Cab Hardware: | Black strap handle, 5½" chassis straps, 16" tilt-back legs, corner protectors, glides |
| Grille: | Silver sparkle grille cloth |
| Logo: | Grille mounted, raised, chrome & black script "Fender" w/ tail |
| Weight: | 53 lbs. (24 Kg) |
| Speakers/Load: | 2 x 12"/4 ohms (8 ohms each in parallel) |
| Speaker Model: | Jensen C12N or Oxford 12L6 |
| Effects: | Reverb, Tremolo |
| Output: | 40 Watts |
| Preamp: | Normal: 7025 Vibrato: 7025, ½ 7025 |
| Power: | 2 x 6L6GC |
| Bias: | Fixed w/ bias adjustment pot. |
| Rectifier: | GZ34 |
| Phase Inverter: | 12AT7 (long tailed) |
| Other: | Reverb Driver: 12AT7 Reverb Recovery: ½ 7025 Tremolo: 12AX7 (photoresistor) |
| Comments: |  |

Black Face Pro Reverb